Research sensors and actuators

1. Sensor: DHT11 (Temperature and Humidity)

- What it does: It measures the ambient temperature and relative humidity of the surrounding air. It is widely used in smart home weather stations and HVAC control systems.

- Electronics/Hardware inside: It contains a capacitive humidity sensor (which uses a moisture-holding substrate between two electrodes) and a thermistor (a resistor whose resistance changes significantly with temperature). It also includes an integrated 8-bit microcontroller to process the raw readings and an internal ADC (Analog-to-Digital Converter) to convert them into a readable signal.

- Is it analog or digital? It is a digital sensor. It uses a custom single-wire serial protocol to send data to the dev kit, which makes it less susceptible to signal noise over long wires compared to analog sensors.

- Units and Range:

  - Humidity: 20% to 90% Relative Humidity (RH) with ±5% accuracy.

  - Temperature: 0 to 50°C with ±2°C accuracy.

2. Actuator: SG90 Micro Servo Motor

- What it does: It provides precise control of angular position (usually between 0 and 180 degrees). In IoT, these are used to move camera mounts, open small latches, move arms in robotics or adjust valves.

- Electronics/Hardware inside: It consists of a DC motor, a gear train (to increase torque), a potentiometer (to track the current position of the shaft), and a control circuit.

- Is it analog or digital? It is controlled via a digital signal called PWM (Pulse Width Modulation).

- Units and Range:

  - Rotation: 0° to 180°.

  - Input Signal: Pulse widths typically ranging from 1ms (for 0°) to 2ms (for 180°), repeating every 20ms (50Hz frequency).